Zhejiang Runhe Chemical New Material Co., Ltd

Add: Rm.1906, Guangli Mansion, No.136, Qingchun Road, Hangzhou, Zhejiang, China

Tel: +86-571-85092338   Fax: +86-571-87246635

Specification

【Product Name】Levelling Agent for Coating RH-T1023

【Component】Polymethyl Alkly-Methyl Phenyl Alkyl polysiloxane

【Instruction】

Organosilicon leveling agent which has defoaming property

It can improve the surface property of solvent and solvent-free coatings, specially for levelling property. Different from mostly silicone auxiliary, it is functioned on defoaming form(No or little foam stailizing) which is depend on polaity of coating system.

RH-1023 increases the smoothness of surface, scratch resistance and anti-adhesive property, prevents Bernard swirl, and improves the orientation of flatting agent and aluminium powder. Even in high solid system which is hard to flatting, it will not cause sparkle. In leveling-coating system, RH-T1023 can make the curtain of coating stable.

RH-T1023 can endure high temperature at 250℃/480°F and it will not low down the interlayer adhesion during the re-coating process.

【Basic Character】

Appearance	: Colorless to amber transparent oily liquid

Non-volatile content: ＞96.0%

【Usage】

The recommended dosage is 0.01～0.3％ of the total formulations

【Package】

200Kg, or 25Kg/Plastic barrel

【Storage】

Transportation as non-dangerous goods

Store in cool and ventilated warehouse. 12 months of storage period.