Zhejiang Runhe Chemical New Material Co., Ltd

Add: Rm.1906, Guangli Mansion, No.136, Qingchun Road, Hangzhou, Zhejiang, China

Tel: +86-571-85092338   Fax: +86-571-87246635

Specification

【Product Name】Levelling Agent for Coating RH-T1033

【Component】Polyether modified polysiloxane

【Instruction】

It can increase the wetting property of the bottom materials by decreasing the surface tension of the coatings. It increases the smoothness of surface, scratch resistance and anti-adhesive property, improves leveling,gloss, and prevents Bernard swirl. Applied as additives in solvent-based, solvent-free based and water-based paint system.

【Basic Character】

Appearance	: Colorless to amber transparent oily liquid

Specific gravity(25℃):1.05±0.05g/ml

Non-volatile content:＞97.0%
Flash point:	＞100℃

Viscosity(25ºC)：800-1500cs

【Usage】

The recommended dosage is 0.05～0.3％ of the total formulations，0.3～1％ in UV and water systems. Generally,it is added as preparing the paints.

【Package】

200Kg, or 25Kg/Plastic barrel

【Storage】

Transportation as non-dangerous goods

Store in cool and ventilated warehouse. 12 months of storage period.